S9_DO Supplementation: Formulae

- Obtain values for the following fields from S3_Aeration Calculator.

- Target AOR, daily (Cell B46 in AOR sheet)

- Target AOR, hourly (Cell B64 in AOR sheet)

- Target Residual DO (Cell C27 in Summary sheet)

- Target Maximum Air Flow (Cell C39 in Summary sheet)

- Target Maximum Power (Cell C42 in Summary sheet)

IF not possible to extract the data, user has to enter the values for the above. 
print(Enter values for the following that were computed by the Aeration calculator)

- User Inputs :

- Current Air Flow supplied (if applicable) in cfm

- Current Power supplied in hp

- Minutes of operation of dosing pump, per hour

- % Deficit in supplying AOR=(Target Max Air Flow - Current Air Flow suppliedTarget Max Air Flow . 100

- Deficit AOR, daily (lbs/day)=% Deficit . Target AOR (daily)

- Deficit AOR, hourly (lbs/hr)=% Deficit . Target AOR (hourly)

- User selects one of the following two chemicals. Corresponding Oxygen value obtained:

| Dosing Chemical (Oxidant) | Amt of Oxygen supplied (lbs) / gal of oxidant |
| --- | --- |
| Hydrogen Peroxide (H2O2) 50% purity | 2.35 |
| Potassium Permanganate (KMnO4) 97% purity | 3.26 |

- Oxidant dosing reqd GPD or gal/d=Deficit AOR,  daily (lbs/d)Amt of Oxygen supplied per gal (lbs/gal)

- Oxidant dosing reqd GPM orgalmin=Deficit AOR,  hourly (lbs/hr)Minutes of operation per hour of dosing pump

- Oxidant dosing reqd mL/min= Oxidant dosing reqd GPM or galmin . 3785.41