Read Me for key understanding (Concept, Target Range, Calculation)

Test for Nutrient Availability - Nitrogen as TIN

- About this test

This document serves as guidance to test for Nitrogen Availability, measured as Total Inorganic Nitrogen (TIN) in the Aeration Basin effluent or outlet end. Essential information is presented in the form of Q&A. In the next page, you will have to enter your measured TIN in mg/L.

- What are the key nutrients that your bugs require?

- Nitrogen (N)

- Phosphorus (P)

- Iron (Fe)

- Why is Nutrient Availability important for your facility’s performance?

- Like humans, your bugs need critical nutrients to perform functions like digestion, reproduction, etc. In the absence of enough nutrients, their metabolism slows down and the BOD goes untreated.

- In case of a nutrient deficiency, a slime tends to form around the floc which interferes with the settling and causes foaming problems when aerated.

- Among higher life-forms, flagellates especially are susceptible to deficiency in nitrogen. A large number of flagellates in an advanced stage of the activated sludge process indicates the presence of a large amount of soluble organic nutrients.

- What are measurement units of Nutrient concentrations?

Nutrient concentrations are measured in mg/L or ppm

- What are monitoring locations for Nutrient concentrations?

Aeration Basin outlet end or effluent

- What is an ideal range of Nitrogen Availability?

Aeration Basin effluent, or near the wastewater near the outlet end must contain 1-2 mg/L of Total Inorganic Nitrogen (Ammonia + Nitrite + Nitrate)

- How do you measure or calculate Nitrogen concentration?

- Test kits and strips are commercially available for measuring different forms of Nitrogen (N) such as Ammonia, Nitrate, Nitrite.

- Lab methods for determining the different forms of Nitrogen, combined to determine Total Inorganic Nitrogen (TIN) are described below.

Total Inorganic Nitrogen =

Ammonia-N (NH3-N) + Nitrate-N (NO4-N) + Nitrite-N (NO3-N)

- Test for Ammonia-N (NH3-N): EPA Method 350.1

A brief summary is given below (Source: EPA). For details, refer to EPA Method 350.1 : Determination of Ammonia-Nitrogen by Semi-Automated Colorimetry.

- Reagents required: 
Ammonia-free water, Permanent ammonia standards, Standard ammonium chloride, Nessler reagent, Sodium hydroxide (12 N), Copper sulfate solution (10%)

- Procedure:

  - Place 100 mL of the sample in a Nessler tube and add 1 mL of Copper sulfate solution

  - Mix by stirring and rotating. Add 1 mL of Sodium hydroxide.

  - Mix again and allow to settle.

  - Pipette a measured portion of the clear supernatant liquor (ideally 25 mL) depending on the ammonia content into a second Nessler tube and dilute to 100 mL with ammonia-free water

  - If permanent ammonia standards are available, go to the next step. Otherwise, prepare temporary standards by adding 0.2, 0.4, 0.6, 0.8, 1.0, 1.4, 1.7, 2.0, 2.5, 3.0 mL of Standard ammonium chloride to Nessler tubes and dilute to 100 mL with ammonia-free water.

  - Add 2 mL of Nessler reagent to the sample, and to each temporary standard (if used)

  - After 10 minutes, compare the colors and record the standard having color nearest to that of the sample.

- Calculations:

  - For permanent standards:
Ammonia-N (mg/L) =

Amount of Ammonia-NItrogen (NH3-N) in permanent standard (mg) x 1000 / Volume of supernatant used in step iv (mL)

  - For temporary standards:
Ammonia-N (mg/L) =

Volume of Standard ammonium chloride (NH4Cl) in temporary standard (mL) x 10 / Volume of supernatant used in step iv (mL)

- Test for Nitrite-N (NO2-N): EPA Method 353.2, 353.3, 1686

A brief summary is given below (Source: EPA). For details, refer to EPA Method 353.2 : Determination of Nitrite/Nitrate-Nitrogen by Automated Colorimetry, or 353.3 : Nitrogen, Nitrate-Nitrite (Spectrophotometric Cadmium Reduction) or Method 1686.

- Reagents required: 
Nitrite-free water, Aluminum hydroxide, Standard sodium nitrite, Sulfanilic acid, A-naphthylamine

- Procedure:

  - If the sample is colored or turbid, clarify 150 mL by adding 2 mL of aluminum hydroxide.

  - Place a measured portion of the filtrate (10-50) mL, depending on the nitrite content, into a 100 mL Nessler tube and make up to the mark with distilled water.

  - If permanent standards are available, go to the next step. Otherwise, prepare temporary standards by adding 0.2, 0.4, 0.6, 0.8, 1.0, 1.4, 1.7, 2.0, 2.5, 3.0 mL of Standard sodium nitrite to Nessler tubes and dilute to 100 mL with nitrite-free water.

  - Add 2 mL of Sulfanilic acid and 2 mL of A-naphthylamine to the sample and to each temporary standard if used.

  - Mix and allow to stand for 10 minutes. Compare the colors and record the standard having color nearest to that of the sample.

- Calculations:

  - For permanent standards:
Nitrite-N (mg/L) =

Amount of Nitrite-NItrogen (NO2-N) in permanent standard (mg) x 1000 / Volume of filtrate used in step ii (mL)

  - For temporary standards:
Nitrite-N (mg/L) =

Volume of Standard sodium nitrite (NaNO2) in temporary standard (mL) x 0.5 / Volume of sample used (mL)

- Test for Nitrate-N (NO3-N): EPA Method 353.2, 353.3, 1686

A brief summary is given below (Source: EPA). For details, refer to EPA Method 353.2 : Determination of Nitrite/Nitrate-Nitrogen by Automated Colorimetry, or 353.3 : Nitrogen, Nitrate-Nitrite (Spectrophotometric Cadmium Reduction) or Method 1686.

You will first need to determine the CHLORIDE concentration of your sample.

- Reagents required: 
Standard Silver nitrate solution (1 mL equivalent of 0.5 mg chloride ion), Potassium chromate indicator (5%), Chloride-free sodium bicarbonate

- Procedure:

    - Pipette 50 or 100 mL of the sample into an Erlenmeyer flask or porcelain casserole depending on the chloride content

    - Add 1 mL Potassium chromate indicator

    - Titrate first to permanent red color with Standard Silver nitrate. If more than 7 or 8 mL of Silver nitrate is required, repeat the procedure using a smaller sample diluted to 50 mL with Chloride-free distilled water

- Calculations:

  - Chloride (mg/L) =

(Volume of Silver nitrate (mL) - Blank) x 500 / 
Volume of sample used (mL)

Now you will need to REMOVE CHLORIDE per the procedure below.

- Reagents required: 
Standard Silver nitrate solution (Prepare this by dissolving 4.40 grams of Silver sulfate, free from nitrate, in distilled water and dilute to 1 L)

- Procedure:

    - Treat 100 mL of the sample with Standard Silver sulfate solution; 1 mL of Standard Silver sulfate solution should be added for each mg of chloride determined previously.

    - Remove the precipitated chloride by either filtration or by centrifugation. Formation of the precipitate may be aided by heating the solution.

Now you can test for NITRATE-N per the procedure below.

- Reagents required: 
Phenoldisulfonic acid, Sodium hydroxide (12 N), Standard nitrate solution

- Procedure:

    - Filter 30 to 35 mL of the sample through filter paper.

    - Evaporate 25 mL of the filtrate to dryness on a water bath (use a smaller amount if the nitrate content is high)

    - Moisten the residue with 1 mL of Phenoldisulfonic acid.

    - Dilute to about 20 mL with distilled water.

    - Add 12 N Sodium hydroxide until the maximum yellow color is developed (not more than 5 to 6 mL of Sodium hydroxide will be required)

    - Filter into a 100 mL Nessler tube and rinse the dish and paper with distilled water. Add the filtered rinsings to the filtrate and make up to the mark with distilled water.

    - If permanent standards are available, proceed to the next step. Otherwise, make up temporary standards by placing 0.2, 0.4, 0.6, 0.8, 1.0, 2.0, 3.0, 4.0 and 5.0 mL of Standard Sodium nitrate solution in 100 mL Nessler tubes and adding 2 mL of 50% Sodium hydroxide.

    - Dilute to the mark with distilled water.

    - Compare the color and record the standard having color nearest to that of the sample.

- Calculations:

  - For permanent standards:
Nitrate-N (mg/L) =

Amount of Nitrate-NItrogen (NO3-N) in permanent standard (mg) x 1000 / Volume of sample used (mL)

  - For temporary standards:
Nitrate-N (mg/L) =

Volume of Standard sodium nitrate (NaNO3) in temporary standard (mL) x 10 / Volume of sample used (mL)

Now combine the three to determine the Total Inorganic Nitrogen (TIN)

Total Inorganic Nitrogen (TIN) = 
Ammonia-N (NH3-N) + Nitrate-N (NO4-N) + Nitrite-N (NO3-N)

- Things to keep in mind to ensure accurate measurement of Nitrogen:

- Ensure you account for all flows entering the Aeration Basins

- Ensure your flowmeters are cleaned and calibrated from time to time

- Further corrective actions, if required, will be specified in the Troubleshooting section

- Next step:

In the next page, you will have to input the average of your measured Nitrogen conc. (in mg/L of Total Inorganic Nitrogen or TIN) at outlet ends or effluent channels all Aeration Basins in service. Jarvis will use this input to test or check if it’s a sign of a problem within your facility.